A copy of our code is pasted below.  Comments are written in red, describing the function of parts of the code.

----------------------------------------------------------------------------------

-- Company:

-- Engineer:

--

-- Create Date: 11/30/2016 12:41:01 PM

-- Design Name:

-- Module Name: main - Behavioral

-- Project Name:

-- Target Devices:

-- Tool Versions:

-- Description:

--

-- Dependencies:

--

-- Revision:

-- Revision 0.01 - File Created

-- Additional Comments:

--

----------------------------------------------------------------------------------

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

-- Uncomment the following library declaration if using

-- arithmetic functions with Signed or Unsigned values

use IEEE.NUMERIC_STD.ALL;

-- Uncomment the following library declaration if instantiating

-- any Xilinx leaf cells in this code.

--library UNISIM;

--use UNISIM.VComponents.all;

entity main is

Port ( ClkIn : in STD_LOGIC;

switch : in STD_LOGIC_VECTOR (15 downto 0);

stop : in STD_LOGIC;

freq : out STD_LOGIC;

lit : out STD_LOGIC_VECTOR (15 downto 0);

gain : out STD_LOGIC;

shutdown : out STD_LOGIC;

an : out STD_LOGIC_VECTOR (3 downto 0);

cat : out STD_LOGIC_VECTOR (6 downto 0));

end main;

architecture Behavioral of main is

--initialize the variables

signal count15, count14, count13, count12, count11, count10, count9, count8, count7, count6, count5, count4, count3, count2, count1, count0 : integer := 0;

signal output : std_logic := '1';

signal counter : integer := 0;

begin

gain <= '1';  --this sets the adapter to a 6 dB gain, a '0' will set it to a 12 dB gain

shutdown <= '1'; --this will restrict current to the speaker if set to a '0'

an <= "0111"; --allows only the first digit of the 7 segment display to light up

process (clkin) begin

if stop = '0' then --makes it so that nothing can occur when the stop button is pushed

if rising_edge(clkin) then --everything runs off of the rising edge of a 100 MHz clock (the system clock)

case (switch) is

when "1000000000000000" => --when SW15 is flipped up

lit <= "1000000000000000"; --LED 15 will illuminate

cat <= "1110010"; --the 7 seg display will display a C

count15 <= count15 + 1; --counter 15 will increment

if count15 = 191110 then --if the counter reaches 191110 (.0019 s)

output <= not output; --output signal will flip high to low or low to high

count15 <= 0; --counter 15 will reset

end if;

when "0100000000000000" => --same thing but new frequencies, LEDs, and digits displayed for switches 14 through 0

lit <= "0100000000000000";

cat <= "1110010";

count14 <= count14 + 1;

if count14 = 180388 then

output <= not output;

count14 <= 0;

end if;

when "0010000000000000" =>

lit <= "0010000000000000";

cat <= "1000010";

count13 <= count13 + 1;

if count13 = 170265 then

output <= not output;

count13 <= 0;

end if;

when "0001000000000000" =>

lit <= "0001000000000000";

cat <= "1000010";

count12 <= count12 + 1;

if count12 = 160705 then

output <= not output;

count12 <= 0;

end if;

when "0000100000000000" =>

lit <= "0000100000000000";

cat <= "0110000";

count11 <= count11 + 1;

if count11 = 151685 then

output <= not output;

count11 <= 0;

end if;

when "0000010000000000" =>

lit <= "0000010000000000";

cat <= "0111000";

count10 <= count10 + 1;

if count10 = 143172 then

output <= not output;

count10 <= 0;

end if;

when "0000001000000000" =>

lit <= "0000001000000000";

cat <= "0111000";

count9 <= count9 + 1;

if count9 = 135139 then

output <= not output;

count9 <= 0;

end if;

when "0000000100000000" =>

lit <= "0000000100000000";

cat <= "0000100";

count8 <= count8 + 1;

if count8 = 127551 then

output <= not output;

count8 <= 0;

end if;

when "0000000010000000" =>

lit <= "0000000010000000";

cat <= "0000100";

count7 <= count7 + 1;

if count7 = 120395 then

output <= not output;

count7 <= 0;

end if;

when "0000000001000000" =>

lit <= "0000000001000000";

cat <= "0001000";

count6 <= count6 + 1;

if count6 = 113636 then

output <= not output;

count6 <= 0;

end if;

when "0000000000100000" =>

lit <= "0000000000100000";

cat <= "0001000";

count5 <= count5 + 1;

if count5 = 107259 then

output <= not output;

count5 <= 0;

end if;

when "0000000000010000" =>

lit <= "0000000000010000";

cat <= "1100000";

count4 <= count4 + 1;

if count4 = 101239 then

output <= not output;

count4 <= 0;

end if;

when "0000000000001000" =>

lit <= "0000000000001000";

cat <= "1110010";

count3 <= count3 + 1;

if count3 = 95560 then

output <= not output;

count3 <= 0;

end if;

when "0000000000000100" =>

lit <= "0000000000000100";

cat <= "1110010";

count2 <= count2 + 1;

if count2 = 90192 then

output <= not output;

count2 <= 0;

end if;

when "0000000000000010" =>

lit <= "0000000000000010";

cat <= "1000010";

count1 <= count1 + 1;

if count1 = 85131 then

output <= not output;

count1 <= 0;

end if;

when "0000000000000001" =>

lit <= "0000000000000001";

cat <= "1000010";

count0 <= count0 + 1;

if count0 = 80354 then

output <= not output;

count0 <= 0;

end if;

when others =>

cat <= "1111111";

lit <= "0000000000000000";

end case;

end if; --for rising edge clock

end if; --for stop input

freq <= output; --freq is set to the output signal used above

end process; --ends process

end Behavioral;

Constraints

set_property PACKAGE_PIN W5 [get_ports ClkIn]

set_property IOSTANDARD LVCMOS33 [get_ports ClkIn]

create_clock -add -name sys_clk_pin -period 10.00 -waveform {0 5} [get_ports ClkIn]

set_property PACKAGE_PIN R2 [get_ports {switch[15]}]

set_property IOSTANDARD LVCMOS33 [get_ports {switch[15]}]

set_property PACKAGE_PIN T1 [get_ports {switch[14]}]

set_property IOSTANDARD LVCMOS33 [get_ports {switch[14]}]

set_property PACKAGE_PIN U1 [get_ports {switch[13]}]

set_property IOSTANDARD LVCMOS33 [get_ports {switch[13]}]

set_property PACKAGE_PIN W2 [get_ports {switch[12]}]

set_property IOSTANDARD LVCMOS33 [get_ports {switch[12]}]

set_property PACKAGE_PIN R3 [get_ports {switch[11]}]

set_property IOSTANDARD LVCMOS33 [get_ports {switch[11]}]

set_property PACKAGE_PIN T2 [get_ports {switch[10]}]

set_property IOSTANDARD LVCMOS33 [get_ports {switch[10]}]

set_property PACKAGE_PIN T3 [get_ports {switch[9]}]

set_property IOSTANDARD LVCMOS33 [get_ports {switch[9]}]

set_property PACKAGE_PIN V2 [get_ports {switch[8]}]

set_property IOSTANDARD LVCMOS33 [get_ports {switch[8]}]

set_property PACKAGE_PIN W13 [get_ports {switch[7]}]

set_property IOSTANDARD LVCMOS33 [get_ports {switch[7]}]

set_property PACKAGE_PIN W14 [get_ports {switch[6]}]

set_property IOSTANDARD LVCMOS33 [get_ports {switch[6]}]

set_property PACKAGE_PIN V15 [get_ports {switch[5]}]

set_property IOSTANDARD LVCMOS33 [get_ports {switch[5]}]

set_property PACKAGE_PIN W15 [get_ports {switch[4]}]

set_property IOSTANDARD LVCMOS33 [get_ports {switch[4]}]

set_property PACKAGE_PIN W17 [get_ports {switch[3]}]

set_property IOSTANDARD LVCMOS33 [get_ports {switch[3]}]

set_property PACKAGE_PIN W16 [get_ports {switch[2]}]

set_property IOSTANDARD LVCMOS33 [get_ports {switch[2]}]

set_property PACKAGE_PIN V16 [get_ports {switch[1]}]

set_property IOSTANDARD LVCMOS33 [get_ports {switch[1]}]

set_property PACKAGE_PIN V17 [get_ports {switch[0]}]

set_property IOSTANDARD LVCMOS33 [get_ports {switch[0]}]

set_property PACKAGE_PIN U18 [get_ports {stop}]

set_property IOSTANDARD LVCMOS33 [get_ports {stop}]

set_property PACKAGE_PIN L1 [get_ports {lit[15]}]

set_property IOSTANDARD LVCMOS33 [get_ports {lit[15]}]

set_property PACKAGE_PIN P1 [get_ports {lit[14]}]

set_property IOSTANDARD LVCMOS33 [get_ports {lit[14]}]

set_property PACKAGE_PIN N3 [get_ports {lit[13]}]

set_property IOSTANDARD LVCMOS33 [get_ports {lit[13]}]

set_property PACKAGE_PIN P3 [get_ports {lit[12]}]

set_property IOSTANDARD LVCMOS33 [get_ports {lit[12]}]

set_property PACKAGE_PIN U3 [get_ports {lit[11]}]

set_property IOSTANDARD LVCMOS33 [get_ports {lit[11]}]

set_property PACKAGE_PIN W3 [get_ports {lit[10]}]

set_property IOSTANDARD LVCMOS33 [get_ports {lit[10]}]

set_property PACKAGE_PIN V3 [get_ports {lit[9]}]

set_property IOSTANDARD LVCMOS33 [get_ports {lit[9]}]

set_property PACKAGE_PIN V13 [get_ports {lit[8]}]

set_property IOSTANDARD LVCMOS33 [get_ports {lit[8]}]

set_property PACKAGE_PIN V14 [get_ports {lit[7]}]

set_property IOSTANDARD LVCMOS33 [get_ports {lit[7]}]

set_property PACKAGE_PIN U14 [get_ports {lit[6]}]

set_property IOSTANDARD LVCMOS33 [get_ports {lit[6]}]

set_property PACKAGE_PIN U15 [get_ports {lit[5]}]

set_property IOSTANDARD LVCMOS33 [get_ports {lit[5]}]

set_property PACKAGE_PIN W18 [get_ports {lit[4]}]

set_property IOSTANDARD LVCMOS33 [get_ports {lit[4]}]

set_property PACKAGE_PIN V19 [get_ports {lit[3]}]

set_property IOSTANDARD LVCMOS33 [get_ports {lit[3]}]

set_property PACKAGE_PIN U19 [get_ports {lit[2]}]

set_property IOSTANDARD LVCMOS33 [get_ports {lit[2]}]

set_property PACKAGE_PIN E19 [get_ports {lit[1]}]

set_property IOSTANDARD LVCMOS33 [get_ports {lit[1]}]

set_property PACKAGE_PIN U16 [get_ports {lit[0]}]

set_property IOSTANDARD LVCMOS33 [get_ports {lit[0]}]

set_property PACKAGE_PIN A14 [get_ports {freq}]

set_property IOSTANDARD LVCMOS33 [get_ports {freq}]

set_property PACKAGE_PIN A16 [get_ports {gain}]

set_property IOSTANDARD LVCMOS33 [get_ports {gain}]

set_property PACKAGE_PIN B16 [get_ports {shutdown}]

set_property IOSTANDARD LVCMOS33 [get_ports {shutdown}]

set_property PACKAGE_PIN W4 [get_ports {an[3]}]

set_property IOSTANDARD LVCMOS33 [get_ports {an[3]}]

set_property PACKAGE_PIN V4 [get_ports {an[2]}]

set_property IOSTANDARD LVCMOS33 [get_ports {an[2]}]

set_property PACKAGE_PIN U4 [get_ports {an[1]}]

set_property IOSTANDARD LVCMOS33 [get_ports {an[1]}]

set_property PACKAGE_PIN U2 [get_ports {an[0]}]

set_property IOSTANDARD LVCMOS33 [get_ports {an[0]}]

set_property PACKAGE_PIN W7 [get_ports {cat[6]}]

set_property IOSTANDARD LVCMOS33 [get_ports {cat[6]}]

set_property PACKAGE_PIN W6 [get_ports {cat[5]}]

set_property IOSTANDARD LVCMOS33 [get_ports {cat[5]}]

set_property PACKAGE_PIN U8 [get_ports {cat[4]}]

set_property IOSTANDARD LVCMOS33 [get_ports {cat[4]}]

set_property PACKAGE_PIN V8 [get_ports {cat[3]}]

set_property IOSTANDARD LVCMOS33 [get_ports {cat[3]}]

set_property PACKAGE_PIN U5 [get_ports {cat[2]}]

set_property IOSTANDARD LVCMOS33 [get_ports {cat[2]}]

set_property PACKAGE_PIN V5 [get_ports {cat[1]}]

set_property IOSTANDARD LVCMOS33 [get_ports {cat[1]}]

set_property PACKAGE_PIN U7 [get_ports {cat[0]}]

set_property IOSTANDARD LVCMOS33 [get_ports {cat[0]}]