#define ENA   14          // Enable/speed motors Right        GPIO14(D5)

#define ENB   12          // Enable/speed motors Left         GPIO12(D6)

#define IN_1  15          // L298N in1 motors Rightx          GPIO15(D8)

#define IN_2  13          // L298N in2 motors Right           GPIO13(D7)

#define IN_3  2           // L298N in3 motors Left            GPIO2(D4)

#define IN_4  0           // L298N in4 motors Left            GPIO0(D3)

#include <ESP8266WiFi.h>

#include <WiFiClient.h>

#include <ESP8266WebServer.h>

String command;             //String to store app command state.

int speedCar = 800;         // 400 - 1023.

int speed_Coeff = 3;

const char* ssid = "RC CAR";

ESP8266WebServer server(80);

void setup() {

pinMode(ENA, OUTPUT);

pinMode(ENB, OUTPUT);

pinMode(IN_1, OUTPUT);

pinMode(IN_2, OUTPUT);

pinMode(IN_3, OUTPUT);

pinMode(IN_4, OUTPUT);

Serial.begin(115200);

// Connecting WiFi

WiFi.mode(WIFI_AP);

WiFi.softAP(ssid);

IPAddress myIP = WiFi.softAPIP();

Serial.print("AP IP address: ");

Serial.println(myIP);

// Starting WEB-server

server.on ( "/", HTTP_handleRoot );

server.onNotFound ( HTTP_handleRoot );

server.begin();

}

void goAhead(){

digitalWrite(IN_1, LOW);

digitalWrite(IN_2, HIGH);

analogWrite(ENA, speedCar);

digitalWrite(IN_3, LOW);

digitalWrite(IN_4, HIGH);

analogWrite(ENB, speedCar);

}

void goBack(){

digitalWrite(IN_1, HIGH);

digitalWrite(IN_2, LOW);

analogWrite(ENA, speedCar);

digitalWrite(IN_3, HIGH);

digitalWrite(IN_4, LOW);

analogWrite(ENB, speedCar);

}

void goRight(){

digitalWrite(IN_1, HIGH);

digitalWrite(IN_2, LOW);

analogWrite(ENA, speedCar);

digitalWrite(IN_3, LOW);

digitalWrite(IN_4, HIGH);

analogWrite(ENB, speedCar);

}

void goLeft(){

digitalWrite(IN_1, LOW);

digitalWrite(IN_2, HIGH);

analogWrite(ENA, speedCar);

digitalWrite(IN_3, HIGH);

digitalWrite(IN_4, LOW);

analogWrite(ENB, speedCar);

}

void goAheadRight(){

digitalWrite(IN_1, LOW);

digitalWrite(IN_2, HIGH);

analogWrite(ENA, speedCar/speed_Coeff);

digitalWrite(IN_3, LOW);

digitalWrite(IN_4, HIGH);

analogWrite(ENB, speedCar);

}

void goAheadLeft(){

digitalWrite(IN_1, LOW);

digitalWrite(IN_2, HIGH);

analogWrite(ENA, speedCar);

digitalWrite(IN_3, LOW);

digitalWrite(IN_4, HIGH);

analogWrite(ENB, speedCar/speed_Coeff);

}

void goBackRight(){

digitalWrite(IN_1, HIGH);

digitalWrite(IN_2, LOW);

analogWrite(ENA, speedCar/speed_Coeff);

digitalWrite(IN_3, HIGH);

digitalWrite(IN_4, LOW);

analogWrite(ENB, speedCar);

}

void goBackLeft(){

digitalWrite(IN_1, HIGH);

digitalWrite(IN_2, LOW);

analogWrite(ENA, speedCar);

digitalWrite(IN_3, HIGH);

digitalWrite(IN_4, LOW);

analogWrite(ENB, speedCar/speed_Coeff);

}

void stopRobot(){

digitalWrite(IN_1, LOW);

digitalWrite(IN_2, LOW);

analogWrite(ENA, speedCar);

digitalWrite(IN_3, LOW);

digitalWrite(IN_4, LOW);

analogWrite(ENB, speedCar);

}

void loop() {

server.handleClient();

command = server.arg("State");

if (command == "F") goAhead();

else if (command == "B") goBack();

else if (command == "L") goLeft();

else if (command == "R") goRight();

else if (command == "I") goAheadRight();

else if (command == "G") goAheadLeft();

else if (command == "J") goBackRight();

else if (command == "H") goBackLeft();

else if (command == "0") speedCar = 400;

else if (command == "1") speedCar = 470;

else if (command == "2") speedCar = 540;

else if (command == "3") speedCar = 610;

else if (command == "4") speedCar = 680;

else if (command == "5") speedCar = 750;

else if (command == "6") speedCar = 820;

else if (command == "7") speedCar = 890;

else if (command == "8") speedCar = 960;

else if (command == "9") speedCar = 1023;

else if (command == "S") stopRobot();

}

void HTTP_handleRoot(void) {

if( server.hasArg("State") ){

Serial.println(server.arg("State"));

}

server.send ( 200, "text/html", "" );

delay(1);

}